Battery Concrete Saw     -    Instructions for Use

FOR INDOOR USE ONLY

PRE-START CHECKS

- Correct blade is fitted for the job.

- Guards are fitted & secure.

- Check blade is securely fastened & sitting square on the shaft (do not over tighten).

- Always use water on Diamond blades.

- Do not use broken or bent blades.

- Beware of loose clothing.

STARTING PROCEDURE

- Place saw on solid ground.

- Make sure batteries are sitting correctly.

- Push start switch to start machine.

- To operate saw push the throttle trigger.

OPERATION

- Always hold saw firmly with both hands.

- Use downward pressure when cutting.

- If engine is revving but blade isn’t turning you are applying too much pressure.

- Release pressure when coming to the end of cut.

STOPPING AND AFTER USE

- Once blade has stopped spinning push START button to turn machine off.

Recharge batteries if necessary

SAFETY PRECAUTIONS

- Do not leave machine running unattended.

- Ensure blade is on squarely and tight.

- Ensure work area is well ventilated.

HAZARDS

- Before cutting check for wiring or pipe work.

- Keep handles free of oil & grease.

- Do not cut products that may contain asbestos.

REQUIRED SAFETY EQUIPMENT