Heater Diesel Fire 25 – Instructions for use

PRE-START CHECKS:

- If using an extension lead (15m or less) it needs to be plugged directly into power.

STARTING PROCEDURE:

- Plug heater into power source – a green light will turn on the side of the machine.

- Push the top of the top switch on the panel IN.

- The heater will run for 20-30 seconds & will fire up on its own when ready.

STOPPING AND AFTER USE:

- Push the bottom of the top switch down so that it sits in the middle.

- The flame goes out & the fan continues to run for 90 seconds cooling the heater.

SAFETY PRECAUTIONS:

- Only use in places free of flammable vapours or high dust content.

- Ensure people & objects are kept a safe distance from the heater (3 metres).

- Keep children & animals away from the heater.

- Unplug heater when not in use.

- Ensure the heater is on a flat, stable surface.

- Only use Diesel.