Large Gas Heater S/S – Instructions for use

PRE-START CHECKS:

- Place heater in suitable position.

- Ensure LPG gas bottle is correctly connected to the heater.

OPERATION:

- Plug into power supply & switch on.

- Attach the heater hose to the gas bottle regulator (left hand thread) & ensure its tight.

- Open valve on gas bottle.

- Depress silver button & hold down.

- Press black button until heater starts whilst maintaining downward pressure on the silver button for 20-30 seconds.

- Release & the heater should be running.

- If flame goes out – repeat the last 2 steps.

STOPPING AND AFTER USE:

- Turn gas off at the bottle.

- Turn off power supply & unplug.

- Unattach the heater from the gas bottle.

SAFETY PRECAUTIONS:

- Do not place anything within a metre of the front of the heater.

- Do not use in enclosed space for extended periods.

REQUIRED SAFETY EQUIPMENT: