PRE-START CHECKs:

- Thoroughly inspect area where the edger is to b used & remove any stones, sticks etc.

- Adjust blade height to suitable height.

Lawn Edger – Instructions for use

OPERATION:

- Switch engine on.

- Slide choke lever up to full choke ON position.

- With one foot on cross brace hold down motor with left hand. With right hand pull the starter grip slowly until you feel it engage -then give it a FAST pull.

- When engine starts to run allow it to fun on half choke for a few seconds to warm up, then push choke off (down) to engine run position.

- Throttle trigger is operated by pushing down interlock with thump & pulling on trigger.

- With both arms extended downwards, hold both handle grips firmly.

- Accelerate engine by pushing down safety interlock with thumb & pulling trigger to full throttle. Blades must be spinning.

- As cutting action begins, move the lawn edger forward so that the blade can cut the edge as you move forward.

STOPPING AND AFTER USE:

- To stop blades rotating release the trigger.

SAFETY PRECAUTIONS:

- Do not lift handles by excessive bending of elbows.

- Do not operate walking backwards.

- Blades should spin under load. If blades rotate slowly, jams or stops in the ground, lower the handles & pull unit back, release trigger & recommence. Re-adjust blade hight.

REQUIRED SAFETY EQUIPMENT: