Skilsaw 8 ¼” Makita – Instructions for use

PRE-START CHECKs:

- When operating a power tool outdoors, use an extension cord suitable for outdoor use.

- Blade depth and bevel adjusting locking levers must be tight and secure.

- Check lower guard for proper closing before each use.

OPERATION:

- Loosen the lever on the depth guide and move the base up or down. At the desired depth of cut, secure the base by tightening the lever.

- For cleaner, safer cuts, set cut depth so that no more than one blade tooth projects below workpiece.

- Plug into power supply.

- Squeeze the trigger and allow saw to reach speed.

- Push the saw forward at a speed so that the blade cuts without slowing. Be sure to move the tool forward in a straight line.

STOPPING AND AFTER USE:

- Before setting the tool down after completing a cut, be sure that the lower guard has closed, and the blade has come to a complete stop.

- Unplug from the power supply.

- Dust machine off.

SAFETY PRECAUTIONS:

- DANGER: Keep hands away from cutting area and the blade. Keep your second hand on auxiliary handle, or motor housing.

- Do not reach underneath the workpiece. The guard cannot protect you from the blade below the workpiece.

- Adjust the cutting depth to the thickness of the workpiece. Less than a full tooth of the blade teeth should be visible below the workpiece.

- Never hold the piece being cut in your hands or across your leg. Secure the workpiece to a stable platform. It is important to support the work properly to minimize body exposure, blade binding, or loss of control.

- Never attempt to saw with the circular saw held upside down in a vise. This is extremely dangerous and can lead to serious accidents.

REQUIRED SAFETY EQUIPMENT: