Waterblaster on Trailer –  Instructions for use

PRE-START CHECKs:

- Ensure adequate water supply is available.

- Avoid kinks in suction & delivery hose.

- Remove trailer unit from tow vehicle before use & place on jockey wheel to keep level.

OPERATION:

- Connect water hose to blaster storage tank.

- Unwind enough pressure hose from trailer unit to move around required area.

- Turn on water supply & allow storage tank to fill.

- Leave water turned on.

- Adjust throttle to half – engage choke for cold start.

- Turn ignition switch & start.

- Return throttle to idle position & allow to warm up.

- Adjust throttle to the full position.

- Depress trigger on gun & commence cleaning.

STOPPING AND AFTER USE:

- Switch off machine.

- Depress trigger on gun to allow the pressure to be released.

- Disconnect all hoses & roll up.

SAFETY PRECAUTIONS:

- Max towing speed is 90km/hr.

- Never spray at people or animals.

- Never hold fingers or hands over high-pressure nozzle.

- Never run the machine out of water.

REQUIRED SAFETY EQUIPMENT: