Chiller Trailer – Instructions for use

PRE-START CHECK:

- Check that the tow coupling is properly secured to the tow ball.

- Tow ball is 1 1/7 inch only.

- Secure safety chain with the “D” shackle to the towing vehicle.

- Attach the trailer light connection lead.

- Stow the jockey wheel/stand into its correct position.

OPERATION:

- Make sure the chiller trailer is on a level surface.

- Ensure the wheels are chocked.

- Wait for 30 minutes after transport before plugging in.

- Plug in the extension cord to the plug that is under the compressor at the front of the chiller box.

- Ensure the power cord is kept out of walkways.

- Chiller should start chilling down within an hour.

SAFETY PRECAUTIONS:

- Maximum towing speed: 90 km/h.

- Do not travel on gravel or uneven roads for long periods of time.