Dyna Drill (Cordless)– Instructions for use

PRE-START CHECK:

- Before inserting the battery cartridge into the tool, check to see that the switch trigger is in the "OFF" position.

- Always check the direction of rotation before operation.

OPERATION:

- To remove the battery cartridge, slide it from the tool while sliding the button on the front of the cartridge.

- To install the battery cartridge, align the tongue on the battery cartridge with the groove in the housing and slip it into place. Insert it all the way until it locks in place with a little click. If you can see the red indicator on the upper side of the button, it is not locked completely.

- Insert the bit into the tool. Turn the bit and push it in until it engages.

- Make sure that the bit is securely held in place by trying to pull it out.

- To remove the bit, pull the chuck cover down all the way and pull the bit out.

- To start the tool, simply pull the switch trigger.

- For drilling in concrete, masonry, etc., depress the lock button and rotate the action mode changing knob to the “rotation with hammering” symbol.

- For drilling in wood, metal or plastic materials, depress the lock button and rotate the action mode changing knob to the “hammering only “symbol.

STOPPING AND AFTER USE:

- Release the switch trigger to stop.

SAFETY PRECAUTIONS:

- Hold the tool firmly with both hands.

- Keep hands away from moving parts.

- Do not touch the bit or parts close to the bit immediately after operation.

- Always switch off the tool before installing or removing of the battery cartridge.

- Use the reversing switch only after the tool comes to a complete stop.

REQUIRED SAFETY EQUIPMENT: